CS3357a Notes

Protocols – Define format, order of messages sent and received among network entities, and actions taken on message transmission, receipt

Network edge:

- Hosts: clients and servers

- Connect to end systems through

  - Residential access nets

  - Institutional access networks (school, company)

  - Mobile access networks

Access networks, physical media: wired, wireless communication links

Network core:

- Interconnected routers

- Network of networks

Digital subscriber line (DSL)

- Use existing telephone line to central office DSLAM (24/2.5 Mbps)

  - Data over DSL phone line goes to Internet

  - Voice over DSL phone line goes to telephone net

Cable network

- Frequency division multiplexing: Different channels transmitted in different frequency bands

- HFC: hybrid fiber coax

  - Asymmetric: up to 30/2 Mbps transmission rate

- Network of cable, fiber attaches homes to ISP router

  - Homes share access network to cable headend unlike DSL, which has dedicated access to central office

Enterprise access networks (Ethernet)

- Typically used in companies, universities, etc.

- 10 Mbps, 100 Mbps, 1 Gbps, 10 Gbps transmission rates

- Today, end systems typically connect into Ethernet switch

Wireless access networks

- Shared wireless access network connects end system to router via base station aka access point

  - Wireless LANs:

    - Within building (100ft)

    - 802.11b/g/n (WiFi): 11, 54, 450 Mbps transmission rate

  - Wire-area wireless access

    - Cellular, 10’s km

    - Between 1 and 10 Mbps

    - 3G, 4G:LTE

Host: sends packets of data

- Takes application message and breaks into smaller chunks, known as packets, of length L bits

- Transmits packet into access network at transmission rate R

  - Link transmission rate, aka link capacity, aka link bandwidth

Packet transmission delay = Time needed to transmit L-bit packet into link = L(bits) / R(bits/sec)

Physical media

- Bit: propagates between transmitter/receiver pairs

- Physical link: what lies between transmitter & receiver

- Guided media: signals propagate in solid media: copper, fiber, coax

- Unguided media: signals propagate freely, eg. Radio

- Twisted pair (TP): Two insulated copper wires (CAT5: 100 Mbps, 1 Gbps Ethernet, CAT6: 10 Gbps)

- Coaxial cable:

  - Two concentric copper conductors

  - Bidirectional

  - Broadband:

    - Multiple channels on cable

    - HFC

- Fiber optic cable

  - Glass fiber carrying light pulses, each pulse a bit

  - High-speed point-to-point transmission (eg. 10’s – 100’s Gbps transmission rate)

  - Low error rate:

    - Repeaters spaced far apart

    - Immune to electromagnetic noise

- Radio

  - Signal carried in electromagnetic spectrum

  - No physical wire

  - Bidirectional

  - Propagation environment effects:

    - Reflection

    - Obstruction by objects

    - Interference

  - Radio link types:

    - Terrestrial microwave

      - Up to 45 Mbps channels

    - LAN (eg WiFi)

      - 54 Mbps

    - Wide-area (eg cellular)

      - 4G cellular: ~10 Mbps

    - Satellite

      - Kbps to 45 Mbps channel (or multiple smaller channels)

      - 270 msec end-end delay

      - Geosynchronous versus low altitude

Network core

- Mesh of interconnected routers

- Packet-switching: hosts break application-layer messages into packets

End-end delay = 2L/R (assuming zero propagation delay)

Routing: determines source-destination route taken by packets through algorithms

Forwarding: move packets from router’s input to appropriate router output

Packet switching allows more users to use network than circuit swiching

- Great for bursts of data, simpler

- Excessive congestion possible

Throughput: rate (bits/time unit) at which bits transferred between sender/receiver

- Instantaneous: rate at given point in time

- Average: rate over longer period of time

Layers: Each layer implements a service via its own internal-layer actions relying on services provided by layer below

- Helps deal with complex systems

- Modularization eases maintenance, updating of system

Internet protocol stack

- Application: supporting network applications

  - FTP, SMTP, HTTP

- Transport: process-process data transfer

  - TCP, UDP

- Network: routing of datagrams from source to destination

  - IP, routing protocols

- Link: data transfer between neighboring network elements

  - Ethernet, 802.11 (WiFi), PPP

- Physical: bits “on the wire”

- Presentation: allow applications to interpret meaning of data (eg encryption, compression, machine-specific conventions)

- Session: synchronization, checkpointing, recovery of data exchange

Cerf and Kahn’s internetworking principles:

- Minimalism, autonomy – no internal changes required to interconnect networks

- Best effort service model

- Stateless routers

- Decentralized control

Define today’s Internet architecture

Chapter 2

Client-server architecture:

Server:

- always-on host

- permanent IP/DNS address

- data centers for scaling

Clients:

- Communicate with server

- May be intermittently connected

- May have dynamic IP addresses

- Do not communicate directly with each other

P2P architecture

- No always-on server

- Arbitrary end systems directly communicate

- Peers request service from other peers, provide service in return to other peers

  - Self scalability – new peers bring service capacity, as well as new service demands

- Peers are intermittently connected and change IP addresses

  - Complex management

Process: program running within a host

Sockets: analogous to door; where processes send/receive messages

To receive messages, process must have identifier that includes both IP address and port numbers associated with process on host.

Internet transport protocols services

- TCP service:

  - Reliable transport between sending and receiving process

  - Flow control: sender won’t overwhelm receiver

  - Congestion control: throttle sender when network overloaded

  - Does not provide: timing, minimum throughput guarantee, security

  - Connection-oriented: setup required between client and server processes

- UDP service:

  - Unreliable data transfer between sending and receiving process

  - Does not provide: reliability, flow control, congestion control, timing, throughput guarantee, security, or connection setup

Securing TCP

- TCP & UDP

  - No encryption

  - Cleartext passwords sent into socket traverse Internet in cleartext

- SSL

  - Provides encrypted TCP connection

  - Data integrity

  - End-point authentication

  - At the app layer: use SSL libraries that “talk” to TCP

  - SSL socket API: cleartext passwords sent into socket traverse Internet encrypted

Non-persistent HTTP: At most one object sent over TCP connection, downloading multiple objects requires multiple connections

Persistent HTTP: multiple objects can be sent over single TCP connection between client and server

RTT: time for a small packet to travel from client to server and back

HTTP response time:

- One RTT to initiate TCP connection

- One RTT for HTTP request and first few bytes of HTTP response to return

- File transmission time

- Non-persistent HTTP response time = 2RTT + file transmission time

Non-persistent HTTP issues:

- Requires 2 RTTs per object

- OS overhead for each TCP connection

- Browsers often open parallel TCP connections to fetch referenced objects

Persistent HTTP:

- Server leaves connection open after sending response

- Subsequent HTTP messages between same client/server sent over open connection

- Client sends requests as soon as it encounters a referenced object

- As little as one RTT for all the referenced objects

Cookies: can be used for

- Authorization

- Shopping carts

- Recommendations

- User session state (Web email)

Electronic mail components:

- User agents

  - “mail reader” that composes, edits, and reads mail messages

  - Outgoing, incoming messages stored on server

- Mail servers

  - Mailbox contains incoming messages for user

  - Message queue of outgoing mail messages

  - SMTP protocol between mail servers to send email messages

    - Client: sending mail server

    - “server”: receiving mail server

- Simple mail transfer protocol: SMTP

  - Uses TCP to reliably transfer email message from client to server, port 25

  - Direct transfer: sending server to receiving server

  - Three phases of transfer

    - Handshaking (greeting)

    - Transfer of messages

    - Closure

  - Command / response interaction

    - Commands: ASCII text

    - Response: status code and phrase

    - Messages must be in 7-bit ASCII

Mail access protocol: retrieval from server

- POP: Post Office Protocol [RFC 1939]: authorization, download

- IMAP: Internet Mail Access Protocol [RFC 1730]: more features, including manipulation of stored messages on server

- HTTP: gmail, Hotmail, Yahoo! Mail, etc.

DNS: hostname to IP address translation

Root name servers: contacted by local name servers that cannot resolve name

Top-level domain (TLD) servers:

- Responsible for com, org, net, edu, aero, jobs, museums, and all top-level country domains (eg uk, fr, ca, jp)

- Network Solutions maintains servers for .com TLD

- Educause for .edu TLD

Authoritative DNS servers:

- Organization’s own DNS server(s), providing authoritative hostname to IP mappings for organization’s named hosts

- Can be maintained by organization or service provider

Local DNS name server

- Does not strictly belong to hierarchy

- Each ISP (residential ISP, company, university) has one

- When host makes DNS query, query is sent to its local DNS server

  - Has local cache of recent name-to-address translation pairs (but may be out of date)

  - Acts as proxy, forwards query into hierarchy

DNS records: Distributed database storing resource records (RR)

- RR format: (name, value, type, ttl)

- Type = A

  - Name is hostname

  - Value is IP address

- Type = NS

  - Name is domain (eg foo.com)

  - Value is hostname of authoritative name server for this domain

- Type = CNAME

  - Name is alias name for some “canonical” (the real) name

  - www.ibm.com is really servereast.backup2.ibm.com

  - Value is canonical name

- Type = MX

  - Value is name of mailserver associated with name

DASH: Dynamic, Adaptive Streaming over HTTP

- Server:

  - Divides video file into multiple chunks

  - Each chunk stored, encoded at different rates

  - Mainifest file: provides URLs for different chunks

- Client:

  - Periodically measures server-to-client bandwidth

  - Consulting manifest, requests one chunk at a time

    - Chooses maximum coding rate sustainable given current bandwidth

    - Can choose different coding rates at different points in time(depending on available bandwidth at the time)

  - Intelligence at client: decides

    - When to request chunk (so that buffer starvation, or overflow does not occur)

    - What encoding rate to request (higher quality when more bandwidth available)

    - Where to request chunk (can request from URL server that is “close” to client or has high available bandwidth)

Chapter 3

Transport services and protocols: provide logical communication between app processes running on different hosts.

Network layer: logical communication between hosts

Transport layer: logical communication between processes

Multiplexing: handle data from multiple sockets, add transport header

Demultiplexing: Use header info to deliver received segments to correct socket

Host receives IP datagrams

Each datagram has source IP address, destination IP address

Each datagram carries one transport-layer segment

Each segment has source, destination port number

Host uses IP addresses & port numbers to direct segment to appropriate socket

- Connectionless demux: must specify destination IP address and port #

- Connection-oriented demux: 4-tuple – Source IP address, port #, dest IP, port #. Each socket identified by its own 4-tuple (web server)

- UDP: User Datagram Protocol

Segments can be lost and delivered out of order

Connectionless: no handshaking, each UDP segment handled independently of others

Add reliability at application layer

- Why is UDP used:

No connection establishment

Simple: no connection state at sender, receiver

Small header size

No congestion control: can blast away as fast as desired

- How to recover from errors:

Acknowledgements (ACKs): receiver explicitly tells sender that pkt received OK

Negative acknowledgements (NAKs): receiver explicitly tells sender that pkt had errors

Sender retransmits pkt on receipt of NAK

- Handling duplicates:

Retransmits current pkt if ACK/NAK corrupted

Adds sequence number to each pkt

Stop and wait: sender sends one packet, then waits for receiver response

- Pipelining: Sender allows multiple, “in-flight”, yet-to-be-acknowledged pkts

Range of sequence numbers must be increased

Buffering at sender and/or receiver

Two generic forms of pipelined protocols: go-Back-N, selective repeat

- Go-back-N:

Sender can have up to N unacked packets in pipeline

Receiver only sends cumulative ack (doesn’t ack packet if there’s a gap)

Sender has timer for oldest unacked packet (when time expires, retransmit all unacked packets)

- Selective Repeat:

Sender can have up to N unacked packets in pipeline

Rcvr sends individual ack for each packet

Sender maintains timer for each unacked packet (when timer expires, retransmit only that unacked packet)

- TCP: Overview

Point-to-point: one sender, one receiver

Reliable, in-order byte stream: no “message boundaries”

Pipelined: TCP congestion and flow control set window size

Full duplex data: bi-directional data flow in same connection

MSS: maximum segment size

Connection-oriented: handshaking inits sender, receiver state before data exchange

Flow controlled: sender will not overwhelm receiver

- Alpha typically = 0.125

- Timeout interval: EstimatedRTT plus “safety margin” (4 * DevRTT)

- TCP creates rdt service on top of IP’s unreliable service

Pipelined segments

Cumulative acks

Single retransmission timer

- Congestion: too many sources sending too much data too fast for the network to handle

Lost packets (buffer overflow at routers)

Long delays (queuing in router buffers)

- TCP congestion control:

Approach: sender increases transmission rate (window size), probing for usable bandwidth, until loss occurs

Additive increase: increase cwnd by 1 MSS every RTT until loss detected

Multiplicative decrease: cut cwnd in half after loss

- Network-assisted congestion control:

Two bits in IP header (ToS field) marked by network router to indicate congestion

Congestion indication carried to receiving host

Receiver (seeing congestion indication in IP datagram) sets ECE bit on receiver-to-sender ACK segment to notify sender of congestion

- Chapter 4

- Network-layer functions:

Forwarding: move packets from router’s input to appropriate router output

Routing: determine route taken by packets from source to destination

- Data plane:

local, per-router function

Determines how datagram arriving on router input port is forwarded to router output port

Forwarding function

- Control plane:

Network-wide logic

Determines how datagram is routed among routers along end-end path from source host to destination host

Two control-plane approaches:

Traditional routing algorithms: implemented in routers

Software-defined networking (SDN): implemented in (remote) servers

- Switching rate: rate at which packets can be transferred from inputs to outputs (memory, bus, crossbar)

- First generation routers:

Traditional computers with switching under direct control of CPU

Packet copied to system’s memory

Speed limited by memory bandwidth (2 bus crossings per datagram)

- Switching via a bus

Datagram from input port memory to output port memory via a shared bus

Bus contention: switching speed limited by bus bandwidth

- Switching via interconnection network

Overcome bus bandwidth limitations

Banyan networks, crossbar, other interconnection nets initially developed to connect processors in multiprocessor

Advanced design: fragmenting datagram into fixed length cells, switch cells through the fabric

- Input port queuing

Fabric slower than input ports combined -> queuing may occur at input queues (buffer overflow)

Head-of-the-Line (HOL) blocking: queued datagram at front of queue prevents others in queue from moving forward

- Scheduling mechanisms

FIFO scheduling

Priority scheduling:

Multiple classes, with different priorities (marking, header info (source/dest, port,etc))

Round Robin (RR)

Multiple classes

Cyclically scan class queues, sending one complete packet from each class (if available)

Weighted Fair Queuing (WFQ)

Generalized Round Robin

Each class gets weighted amount of service in each cycle

- Interface: Connection between host/router and physical link

- Subnet: device interfaces with the same subnet part of IP address (reach without router)

- CIDR: Classless InterDomain Rounting

Subnet portion of address of arbitrary length

Address format: a.b.c.d/x, where x is # bits in subnet portion of address

- DHCP: Dynamic Host Configuration Protocol: dynamically get address from a server (“plug-and-play”)

- Hierarchical addressing: allows efficient advertisement of routing information

- ICANN: Internet Corporation for Assigned Names and Numbers

Allocates addresses

Manages DNS

Assigns domain names, resolves disputes

- NAT: network address translation: Local network uses just one IP address as far as outside world is concerned:

Range of addresses not needed from ISP: just one IP address for all devices

Can change addresses of devices in local network without notifying outside world

Can change ISP without changing addresses of devices in local network

Devices inside local net not explicitly addressable, visible by outside world (a security plus)

- NAT router must:

Outgoing datagrams: replace (source IP address, port#) of every outgoing datagram to (NAT IP address, new port #), remote clients/servers will respond using (NAT IP address, new port #) as destination addr

Remember (in NAT translation table) every (source IP address, port #) to (NAT IP address, new port #) translation pair

Incoming datagrams: replace (NAT IP address, new port #) in dest fields of every incoming datagram with corresponding (source IP address, port #) stored in NAT table

- NAT:

16-bit port-number field:

60 000 simultaneous connections with a single LAN-side address

Controversial:

Routers should only process up to layer 3

Address shortage should be solved by IPv6

Violates end-to-end argument

NAT possibility must be considered by app designers, eg P2P applications

NAT traversal: what if client wants to connect to server behind NAT?

- IPv6 motivation:

32-bit address space soon to be completely allocated

Header format helps speed processing/forwarding

Header changes to facilitate QoS

- IPv6 datagram format:

Fixed-length 40 byte header

No fragmentation allowed