CS3357a Notes

Protocols – Define format, order of messages sent and received among network entities, and actions taken on message transmission, receipt

Network edge:

- Hosts: clients and servers

- Connect to end systems through

  - Residential access nets

  - Institutional access networks (school, company)

  - Mobile access networks

Access networks, physical media: wired, wireless communication links

Network core:

- Interconnected routers

- Network of networks

Digital subscriber line (DSL)

- Use existing telephone line to central office DSLAM (24/2.5 Mbps)

  - Data over DSL phone line goes to Internet

  - Voice over DSL phone line goes to telephone net

Cable network

- Frequency division multiplexing: Different channels transmitted in different frequency bands

- HFC: hybrid fiber coax

  - Asymmetric: up to 30/2 Mbps transmission rate

- Network of cable, fiber attaches homes to ISP router

  - Homes share access network to cable headend unlike DSL, which has dedicated access to central office

Enterprise access networks (Ethernet)

- Typically used in companies, universities, etc.

- 10 Mbps, 100 Mbps, 1 Gbps, 10 Gbps transmission rates

- Today, end systems typically connect into Ethernet switch

Wireless access networks

- Shared wireless access network connects end system to router via base station aka access point

  - Wireless LANs:

    - Within building (100ft)

    - 802.11b/g/n (WiFi): 11, 54, 450 Mbps transmission rate

  - Wire-area wireless access

    - Cellular, 10’s km

    - Between 1 and 10 Mbps

    - 3G, 4G:LTE

Host: sends packets of data

- Takes application message and breaks into smaller chunks, known as packets, of length L bits

- Transmits packet into access network at transmission rate R

  - Link transmission rate, aka link capacity, aka link bandwidth

Packet transmission delay = Time needed to transmit L-bit packet into link = L(bits) / R(bits/sec)

Physical media

- Bit: propagates between transmitter/receiver pairs

- Physical link: what lies between transmitter & receiver

- Guided media: signals propagate in solid media: copper, fiber, coax

- Unguided media: signals propagate freely, eg. Radio

- Twisted pair (TP): Two insulated copper wires (CAT5: 100 Mbps, 1 Gbps Ethernet, CAT6: 10 Gbps)

- Coaxial cable:

  - Two concentric copper conductors

  - Bidirectional

  - Broadband:

    - Multiple channels on cable

    - HFC

- Fiber optic cable

  - Glass fiber carrying light pulses, each pulse a bit

  - High-speed point-to-point transmission (eg. 10’s – 100’s Gbps transmission rate)

  - Low error rate:

    - Repeaters spaced far apart

    - Immune to electromagnetic noise

- Radio

  - Signal carried in electromagnetic spectrum

  - No physical wire

  - Bidirectional

  - Propagation environment effects:

    - Reflection

    - Obstruction by objects

    - Interference

  - Radio link types:

    - Terrestrial microwave

      - Up to 45 Mbps channels

    - LAN (eg WiFi)

      - 54 Mbps

    - Wide-area (eg cellular)

      - 4G cellular: ~10 Mbps

    - Satellite

      - Kbps to 45 Mbps channel (or multiple smaller channels)

      - 270 msec end-end delay

      - Geosynchronous versus low altitude

Network core

- Mesh of interconnected routers

- Packet-switching: hosts break application-layer messages into packets

End-end delay = 2L/R (assuming zero propagation delay)

Routing: determines source-destination route taken by packets through algorithms

Forwarding: move packets from router’s input to appropriate router output

Packet switching allows more users to use network than circuit swiching

- Great for bursts of data, simpler

- Excessive congestion possible

Throughput: rate (bits/time unit) at which bits transferred between sender/receiver

- Instantaneous: rate at given point in time

- Average: rate over longer period of time

Layers: Each layer implements a service via its own internal-layer actions relying on services provided by layer below

- Helps deal with complex systems

- Modularization eases maintenance, updating of system

Internet protocol stack

- Application: supporting network applications

  - FTP, SMTP, HTTP

- Transport: process-process data transfer

  - TCP, UDP

- Network: routing of datagrams from source to destination

  - IP, routing protocols

- Link: data transfer between neighboring network elements

  - Ethernet, 802.11 (WiFi), PPP

- Physical: bits “on the wire”

- Presentation: allow applications to interpret meaning of data (eg encryption, compression, machine-specific conventions)

- Session: synchronization, checkpointing, recovery of data exchange

Cerf and Kahn’s internetworking principles:

- Minimalism, autonomy – no internal changes required to interconnect networks

- Best effort service model

- Stateless routers

- Decentralized control

Define today’s Internet architecture

Chapter 2

Client-server architecture:

Server:

- always-on host

- permanent IP/DNS address

- data centers for scaling

Clients:

- Communicate with server

- May be intermittently connected

- May have dynamic IP addresses

- Do not communicate directly with each other

P2P architecture

- No always-on server

- Arbitrary end systems directly communicate

- Peers request service from other peers, provide service in return to other peers

  - Self scalability – new peers bring service capacity, as well as new service demands

- Peers are intermittently connected and change IP addresses

  - Complex management

Process: program running within a host

Sockets: analogous to door; where processes send/receive messages

To receive messages, process must have identifier that includes both IP address and port numbers associated with process on host.

Internet transport protocols services

- TCP service:

  - Reliable transport between sending and receiving process

  - Flow control: sender won’t overwhelm receiver

  - Congestion control: throttle sender when network overloaded

  - Does not provide: timing, minimum throughput guarantee, security

  - Connection-oriented: setup required between client and server processes

- UDP service:

  - Unreliable data transfer between sending and receiving process

  - Does not provide: reliability, flow control, congestion control, timing, throughput guarantee, security, or connection setup

Securing TCP

- TCP & UDP

  - No encryption

  - Cleartext passwords sent into socket traverse Internet in cleartext

- SSL

  - Provides encrypted TCP connection

  - Data integrity

  - End-point authentication

  - At the app layer: use SSL libraries that “talk” to TCP

  - SSL socket API: cleartext passwords sent into socket traverse Internet encrypted

Non-persistent HTTP: At most one object sent over TCP connection, downloading multiple objects requires multiple connections

Persistent HTTP: multiple objects can be sent over single TCP connection between client and server

RTT: time for a small packet to travel from client to server and back

HTTP response time:

- One RTT to initiate TCP connection

- One RTT for HTTP request and first few bytes of HTTP response to return

- File transmission time

- Non-persistent HTTP response time = 2RTT + file transmission time

Non-persistent HTTP issues:

- Requires 2 RTTs per object

- OS overhead for each TCP connection

- Browsers often open parallel TCP connections to fetch referenced objects

Persistent HTTP:

- Server leaves connection open after sending response

- Subsequent HTTP messages between same client/server sent over open connection

- Client sends requests as soon as it encounters a referenced object

- As little as one RTT for all the referenced objects

Cookies: can be used for

- Authorization

- Shopping carts

- Recommendations

- User session state (Web email)

Electronic mail components:

- User agents

  - “mail reader” that composes, edits, and reads mail messages

  - Outgoing, incoming messages stored on server

- Mail servers

  - Mailbox contains incoming messages for user

  - Message queue of outgoing mail messages

  - SMTP protocol between mail servers to send email messages

    - Client: sending mail server

    - “server”: receiving mail server

- Simple mail transfer protocol: SMTP

  - Uses TCP to reliably transfer email message from client to server, port 25

  - Direct transfer: sending server to receiving server

  - Three phases of transfer

    - Handshaking (greeting)

    - Transfer of messages

    - Closure

  - Command / response interaction

    - Commands: ASCII text

    - Response: status code and phrase

    - Messages must be in 7-bit ASCII

Mail access protocol: retrieval from server

- POP: Post Office Protocol [RFC 1939]: authorization, download

- IMAP: Internet Mail Access Protocol [RFC 1730]: more features, including manipulation of stored messages on server

- HTTP: gmail, Hotmail, Yahoo! Mail, etc.

DNS: hostname to IP address translation

Root name servers: contacted by local name servers that cannot resolve name

Top-level domain (TLD) servers:

- Responsible for com, org, net, edu, aero, jobs, museums, and all top-level country domains (eg uk, fr, ca, jp)

- Network Solutions maintains servers for .com TLD

- Educause for .edu TLD

Authoritative DNS servers:

- Organization’s own DNS server(s), providing authoritative hostname to IP mappings for organization’s named hosts

- Can be maintained by organization or service provider

Local DNS name server

- Does not strictly belong to hierarchy

- Each ISP (residential ISP, company, university) has one

- When host makes DNS query, query is sent to its local DNS server

  - Has local cache of recent name-to-address translation pairs (but may be out of date)

  - Acts as proxy, forwards query into hierarchy

DNS records: Distributed database storing resource records (RR)

- RR format: (name, value, type, ttl)

- Type = A

  - Name is hostname

  - Value is IP address

- Type = NS

  - Name is domain (eg foo.com)

  - Value is hostname of authoritative name server for this domain

- Type = CNAME

  - Name is alias name for some “canonical” (the real) name

  - www.ibm.com is really servereast.backup2.ibm.com

  - Value is canonical name

- Type = MX

  - Value is name of mailserver associated with name

DASH: Dynamic, Adaptive Streaming over HTTP

- Server:

  - Divides video file into multiple chunks

  - Each chunk stored, encoded at different rates

  - Mainifest file: provides URLs for different chunks

- Client:

  - Periodically measures server-to-client bandwidth

  - Consulting manifest, requests one chunk at a time

    - Chooses maximum coding rate sustainable given current bandwidth

    - Can choose different coding rates at different points in time(depending on available bandwidth at the time)

  - Intelligence at client: decides

    - When to request chunk (so that buffer starvation, or overflow does not occur)

    - What encoding rate to request (higher quality when more bandwidth available)

    - Where to request chunk (can request from URL server that is “close” to client or has high available bandwidth)